# Maintenance Guide

For Aboveground Pools

Includes:

General Operations								page 2

Backwashing									page 3

Vacuuming									page 4

Water Testing/Pool Maintenance						page 5

Pool Chemicals								page 6-8

Troubleshooting								page 9

3117 S. Campbell Ave Springfield, MO 65807

(417) 883-8827

## General Operations & Maintenance

Skimmer and pump baskets should be checked and/or emptied every 3-4 days. If you have a lot of trees, you may want to do this daily during the summer.

The water should always be halfway up on the skimmer mouth.

Never turn pool lights on when pool is drained. Lights should be turned on only when they are submerged.

The pump and filter should never run without water in both. If the pressure gauge indicates zero pressure, this indicates that there is no water moving through the pump and filter, and it is necessary to prime the pump.

To prime the pump:

- Turn off the pump.

- Close any ball valves (only if you have rigid PVC plumbing).

- Remove lid or pump strainer pot.

- Take garden hose and fill pot with water.

- Replace and tighten lid.

- Open main drain ball valve (only if you have rigid PVC plumbing).

- Turn on pump and wait until pressure builds to normal reading.

(If pump does not prime after about a minute, shut off and repeat above steps)

- Open skimmer ball valve slowly (only if you have rigid PVC plumbing).

Never add chemicals to pool while swimmers are in the water.

Absolutely do not mix chemicals.

Pool chemicals can be dangerous. Read all labels and use products according to directions. Store chemicals in a cool, dry place.

Backwashing (Cleaning Filter)

Backwashing your filter is very important, as it reverses the flow of water through the filter sand, rinsing the debris out of it. The pressure gauge for the filter should be positioned somewhere near the valve handle, and should be checked at least once a week.

Normal operating pressure will vary widely from filter to filter. To find what your norm is, backwash the filter thoroughly (about 2-3 minutes), then record below what the psi reads on your gauge when you start the filter back up.

Normal psi  _____________

The filter should be backwashed when the pressure reaches 8 – 10 psi above normal, or once a week, whichever comes first. Sand in the filter should be replaced every 3 – 5 years on a chlorine system, and every year on a Softswim system.

Filters will not be damaged by backwashing more frequently. If you are having problems with hazy or green water, backwash the filter daily. This will help get rid of bacteria and algae in the water faster.

To backwash sand and DE filters with multi-port valves:

- Turn off pump.

- Turn valve handle to backwash.

- Roll out backwash hose.

- Turn on pump.

- Allow filter to backwash until water clears in sight glass (45-60 seconds).

- After water clears, turn pump off.

- Turn valve handle to rinse.

- Turn on pump.

- Allow filter to rinse for 30 seconds.

- Turn off pump.

- Return handle to filter position.

- Turn on pump.

NOTE: Repositioning multi-port valve while pump is in operation may cause damage to the filtering system. Always shut pump off first.

All ball valves should be open while backwashing.

## Manual Vacuuming

If you have an automatic cleaner, please skip to step 19. Refer to your cleaner’s instruction manual for details on how to install.

- Keep valve handle on filter position.

- Close main drain ball valve (for rigid PVC plumbing only).

- Attach vacuum head to pole.

- Attach vacuum hose to vacuum head.

- Place head and hose in the water.

- Feed vacuum hose straight down into the water, allowing hose to fill with water.

- Keeping free end of vacuum hose in the water, move to the skimmer and attach hose to vac plate set over skimmer basket.

Doughboy pools only: Take skimmer lid off of top of skimmer, angle and place down snugly over skimmer basket. Once hose has filled, insert vacuum adapter in the end of hose, and plug other end of adapter into the hole beneath the skimmer.

- Vacuum pool.

- After vacuuming is finished, remove vacuum hose from skimmer.

- Turn off pump.

- Shut off skimmer ball valve (rigid PVC plumbing only).

- Remove skimmer basket and empty. Check pump strainer basket and empty if needed.

- Replace basket and lid (for Doughboy, replace lid in normal position on top of skimmer.)

- Open all ball valves (rigid PVC plumbing only).

- Turn on pump and allow pressure to build.

- Backwash the filter. (see previous page)

- Remove vacuum equipment from the pool.

- For proper vacuuming suction, it is important that the filter is clean prior to vacuuming. If necessary, backwash before vacuuming also. Check and make sure pressure gauge is reading within normal psi before starting to vacuum.

- Once a week the pool should be brushed down with a vinyl bristled pool brush before vacuuming. This will loosen any molecules that might be stuck to the walls or floor.

## Water Testing and Maintenance

Pool water should be tested with a test kit or strips at least once a week.

When testing with test strips, take one strip out of bottle, and screw lid back onto bottle. (This eliminates the chance of leaning over the pool with the bottle in your hand and dumping all 50 strips in the pool!)

Dip your strip into the water, away from the jet return, and immediately bring strip back out of water and hold level for 15 seconds. After exactly 15 seconds, match colors on strip to colors on back of bottle.

Adjust pH and total alkalinity if necessary, according to directions in Chemicals section. If chlorine is 5 ppm or higher, no shock treatment needs to be added for that week.

Recommended levels for pool:

Free Chlorine		1.0 – 3.0 ppm

PH				7.2 – 7.6

Total Alkalinity		125 – 150 ppm

If testing with a kit, make sure you test for a free chlorine level. Refer to the instructions on your kit to see whether your test is for total or free chlorine.

Bring in a 12 oz. water sample for free computerized analysis at least once a month during the summer season.

### Maintenance

The pool walls need to be brushed once a week. Use the vinyl bristled brush on the end of your telescoping pole, or get in and brush while you swim!

Vacuum pool as needed to keep floor clean. The less debris in the water, the less chemicals you’ll use. An automatic pool cleaner comes in handy here!

If you have a solar blanket, make sure you take it off the pool every 2 – 3 days so the pool can breathe, and gas off bacteria that has been killed.

The best thing you can do to keep your pool clear is run the pump and filter system 24 hours a day, every day. Filtration is very important to water clarity. Follow the directions for the 3 Step Program below, and you will have a sparkling pool all summer long! (And that’s what you bought it for, right?)

## Pool Chemicals

Basic chemicals you will need to have on hand:

### Chemical			What It Does

Step 1: Smart Sticks	Sanitizer for pool, added to skimmer weekly

Step 2: Smart Shock	Oxidizes bacteria out of water, added weekly

Step 3: Back Up		Helps prevent algae, added weekly

Lo N Slo			Ph and Total Alkalinity Decreaser

Balance Pak 200		Ph Increaser

Balance Pak 100		Total Alkalinity Increaser

Stabilizer 			Keeps chlorine from evaporating under the sun

Off the Wall			Pool wall and skimmer cleaner

Test Strips			Test for free chlorine, ph, total alkalinity

#### Spring Start Up and/or Fresh Fill

First: Above ground pools should never be completely drained, unless the liner is being replaced. The pool needs water to support the sidewalls, and keep the vinyl liner soft and pliant. Pools can be partially drained if fresh water is needed.

Step 1: If more than 10” of fresh water needs to be added upon start up, add Stabilizer 100 first. Use 1 pound for every 3,000 gallons of water added. Always add Stabilizer 100 through the skimmer with the pump running, or pre-dissolve for several days before broadcasting solution into pool. For exact dosage, bring water sample into store for professional analysis.

Step 2: If water is clear, follow these instructions:

- Add Smart Sticks to the skimmer basket. Add 1 stick per 5,000 gallons.

- Add Burn Out 35 directly to pool with pump running. Use 1 bag per 6,000 gallons.

- Add Back Up algaecide directly to pool. Use 1 ounce per 1,000 gallons.

If water is cloudy or green, follow these instructions:

- Add Smart Sticks to the skimmer basket. Add 1 stick per 5,000 gallons.

- Add Burn Out 35 or Super Soluble directly to pool with pump running. Use 1 bag per 3,000 gallons for initial treatment. Test for free chlorine level with test strip every 12 – 24 hours. Repeat this shock treatment if chlorine level shows less than 2 ppm, for as long as necessary to establish chlorine level of at least 3 ppm. When chlorine level stays at 3 ppm for more than 24 hours, pool will start to clear.

- Add Back Up algaecide if water is cloudy. Use 1 ounce per 1,000 gallons. Add Banish algaecide is water is green. Use 1 ounce per 1,000 gallons. Do this step with first shock treatment.

ALWAYS run pump 24 hours per day if water is not clear. This will help water clear faster. Backwash every 24-48 hours as needed to relieve filter pressure.

## 3 Step Program for Weekly Maintenance

Step 1:  Add Smart Sticks to big skimmer basket. Usage:

Pools 12’ to 21’	1 stick per week

Pools 24’ to 28’	2 sticks per week

Pools 30’ and up	3 sticks per week

If sticks are not dissolving at this rate, run pump longer per day. (Ideally, pump should run 24 hours per day, but never less than 12 hours per day).

Chlorine sticks can also be added to an automatic chlorinator, if you have one  (Silk Sticks are best for this method).

To refill chlorinator:

- Turn off pump.

- Remove lid from chlorinator.

- Insert Silk Sticks, as many as needed to fill chamber.

- Replace lid on chlorinator. Make sure o-ring is in place under the lid.

- Turn on pump.

- Adjust metering valve if necessary.

Never put chemicals in the chlorinator other than the ones recommended by your pool dealer. Read danger tag on chlorinator.

Step 2: Add Smart Shock directly to pool water with pump running. Usage:

Pools 12’ to 18’ 	½ bag per week

Pools 21’ to 24’ 	1 bag per week

Pools 28’ to 30’	2 bags per week

Pools 32’ and up	3 bags per week

Deep ends will add extra water to these dosages. If you have a deep end, add an extra ½ bag per week for pools up to 24’, and 1 extra bag to pools 28’ and up.

If pool water is cloudy or green, or no free chlorine level is present, you should add Burn Out 35 or Super Soluble instead of Smart Shock. Bring a sample into store for accurate analysis as soon as possible.

Step 3: Add Back Up algaecide after you shock the pool (at least 2 hours after). 		Pools 12’ to 18’ 	Add 1 ounces per week

Pools 21 to 24’	Add 2 ounces per week

Pools 28’ to 30’	Add 3 ounces per week

Pools 32’ and up	Add 4 ounces per week

For pools with deep ends, add 1 extra ounce to normal weekly dosage.

Balancers: Test water with test strips at least once a week. Dip strip into water, and hold strip level for exactly 15 seconds. This is very important to get accurate levels. Adjust pH and total alkalinity if necessary. Lo N Slo and Balance Pak 200 should only be added when the test strip indicates the pH is off. These products will raise or lower pH. Add products per directions on back of bottle, or bring a water sample in to the store to be tested.

Stabilizer should be added to the pool whenever a partial drain or fresh fill is required. After water is brought back up to mid-skimmer level, refer to instructions on the bottle to add stabilizer. Only water loss can decrease stabilizer levels.

NOTE: This product is very slow to dissolve. If possible, predissolve before adding to water, or add product through slowly through skimmer.

If there is ever a problem with your water, or if you just want to check yourself, we test water every day for FREE! Bring a 12 oz sample in, and we’ll let you know how you’re doing.

STEP 4: MOST IMPORTANT. For your own sanity and overall well-being, get into the pool and swim around at least 30 minutes each day. Get the kids, have a huge splash fight. You’ll remember why you wanted this pool in the first place!

## Troubleshooting Tips

Most problems with the pool can be easily fixed yourself. Some common problems and solutions are:

Problem: Pool leaking around equipment area.

Solution: Tighten clamps on all connecting hoses. Make sure drain

plugs are tightly installed on sand filter tank and pump.

Make sure pump lid is on tight and o-ring is seated correctly.

Problem: Pool filter pressure running too high, water pressure weak

at return jet.

Solution: Filter needs to be backwashed. If you have recently backwashed

already, try backwashing for longer, 2-3 minutes. Remember, sand

also needs to be replaced every 3 – 5 years.

Problem: Pool filter pressure low or zero, and/or bubbles coming out of

return jet.

Solution: Make sure water level in pool is at mid-skimmer. Make sure skimmer

itself is full of water. Make sure pump basket is full of water, and water

is circulating through system. If you can see water churning in pump

basket, there is air getting into system. Check hoses for pinholes,

tighten clamps on hose from skimmer down to pump. Also check

pump lid and o-ring for cracks, lid or o-ring may need to be replaced or

lubricated. Make sure pump lid is tightened securely.

Problem: Pool water making loud surging or gurgling noise.

Solution: Water level in pool is probably low, add water to pool until it hits

mid-skimmer level.